MBSD Lab #2 A.Y. 2023/24

# Purposes

- Implement the “one pedal controller” as a Simulink model.

To implement the system, follow the description already provided in the Laboratory 1 document considering also eventual assumptions already stated in this latter document.

The safety mechanisms are not required in this laboratory.

The Simulink project has to be split into 3 files:

- Harness.slx, containing reference models for the controller and plant and test stimuli generation

- Controller.slx, containing the controller (to be developed)

- Plant.slx, containing the car longitudinal physical model.

The longitudinal physical model and the test stimuli generators shall demonstrate the effectiveness of the implemented controller with respect to the expected functionalities.

Consider the plant model description as an example of the comments to be inserted in the Controller Software Unit  description report.

Templates of these files are available alongside this document.

The deliverable, composed of the 3 Simulink models and a PDF file obtained by filling the following pages of this document (please delete this first page), has to be provided as a .ZIP file up to May 24th at 23:59 through “Consegna Elaborati” of the Portale della Didattica. It shall also contain a brief report explaining the design of the controller, using the following template. It is sufficient that only one of the group members uploads it.

# Model-Based Software Design, A.Y. 2023/24

# Laboratory 2 Report

## Components of the working group (max 2 people)

- Matteo Gravagnone, s319634

- Danilo Guglielmi, s318083

## External interfaces of the plant

| Name | Direction | Type |
| --- | --- | --- |
| TorqueRequest_Nm | Input | CAN |
| Vehicle_Speed_km_h | Output | CAN |
| AutomaticTransmissionSelectorState | Input (from the driver to the controller) | CAN {P, R, N, D, P} |
| Selected mode/errors | Output (to the driver) | CAN |

## Equations of the plant

The plant considered in this model is the so-called Vehicle Longitudinal Dynamics.

Considering:

- vt the vehicle acceleration, expressed in [m/s2]

- vt the vehicle longitudinal speed, expressed in [m/s]

- m the vehicle mass, expressed in [kg]

- Fx(t) the longitudinal force applied to the vehicle center of gravity, expressed in [N]

- Fs(t) the longitudinal force applied to the wheel on the terrain, expressed in [N]

- Fd(v) the longitudinal force applied to the vehicle center of gravity due to the frictions with air and terrain, expressed in [N]

- I the moment of inertia of each one of the wheels, expressed in [kg⋅m2]

- r the radius of the wheel, expressed in [m]

- ω(t) is the angular speed of the wheel, expressed in [rad/s]

- ωet is the angular speed of the engine/electrical motor, expressed in [rad/s]

- ωeRPMt=ωet⋅602π is the angular speed of the engine/ electrical motor, expressed in [revolutions per minutes]

- ω(t) is the angular acceleration of the wheel, expressed in [rad/s2]

- S is the frontal surface of the car, expressed in [m2]

- cx=0.3 is the automobile drag coefficient

- ρ is the average density of air at sea level in standard conditions 
ρ= 1.25 kg/m3

- ig is the gearbox reduction ratio

- if is the final drive reduction ratio

- it=ig⋅if is the total power train reduction ratio.

An extremely simplified model can be obtained as follow:

vt=Fxt-Fdvm     (1)

where vt is the vehicle acceleration, m is its mass, Fx(t) is the longitudinal force applied to its center of gravity by the effects of the torque applied on the wheels, and Fd(v) is the sum of the friction forces on the vehicle due to wheel-terrain and vehicle-air interactions.

Considering that the torque is equally split between the two wheels (valid only on straight tracks)

Tt-2⋅Fst⋅r= 2I⋅ωt     (2)

the absence of slipping:

2⋅Fst⋅r=Fx(t)vt=2 π⋅r⋅ ω(t)

and considering the moment of inertia of the wheels I=0, we can define the following equation, given that Tt=2⋅Fs(t)⋅r  Fst=T(t)2⋅r.

The drag force that limits the maximum speed of the vehicle is equal to:

Fdvt=Xair⋅vt2+Xtyres⋅v(t)     (3)

where:

Xair=12⋅S⋅cx⋅ρ     (4)

and, as usually modeled:

Xtyres|Xtyres50kmh=Xair50kmh→ Xtyres=Xair⋅503.6     (5)

By substituting the (2) equation in (1), and by integrating both sides, we obtain:

vt=1m0tFxt-Fdvtdt=1m0t2⋅Fst- Fdvtdt=

=1m0tTtr- Fdvt dt     (6)

and, by substituting (3) in (6):

vt=1m0tTtr- Xair⋅vt2-Xtyres⋅v(t) dt     (7)

Remember that the integrator block of Simulink requires an initial condition corresponding to the vehicle's longitudinal speed at the beginning of the simulation, v0. A possible configuration of the integration block is shown in Figure 2.

During the model development, put all the needed gain to obtain as an output of the physical model a speed expressed in km/h.

To simulate the slope θ of the terrain, it is possible to add the gravity force Fg(θ) as follows:

vt=1m0tTtr- Xair⋅vt2-Xtyres⋅|vt|⋅dt+m⋅g⋅sin⁡(θ)     (8)

With g=9.81 the gravity acceleration on Earth.

Reasonable values for an electric compact car can be:

- m=1600 kg

- r=0.3 m

- The torque T (at the wheel) can vary in the range -60;960 Nm

- S=3.5 m2

- cx=0.3

Figure 1 Graph showing drag forces of tires (in orange) and air (in blue) at various speeds. It is possible to observe that, as imposed in equation (5), Xtyre=Xair at 50 km/h. Below this speed, the tire drag is dominant, after that, the air drag is dominant. Moreover, it is possible to see the top speed of the car (around 230 km/h) when Fx= Tr=Xtyre+Xair, with Fx=3200 N.

With those values, the top speed on level ground reachable by the car is about 230 km/h, where the drag forces equal the traction force (3200 N).

Considering the reverse direction, the maximum speed reachable with a limitation of -60 Nm is about 45 km/h.

Figure 2 Settings window for the Integrator block of Simulink

Use these values (with a certain tolerance, for example, 10 %) to saturate the integrator block.

To make the model more realistic, it is possible to compute the torque request at the engine/motor. A typical ratio value for transmission of an electric car with a single gear can be around it=12.

All the initialization parameters of the model are automatically loaded model by a callback of the function init_fn as shown in Figure 3.

Figure 3 init_fn callback configuration in the harness model properties.

## Description of the whole system

Draw the I/O block diagram of the plant and the controller, showing how they interact.

1 - Controller and plant

2 - Controller

# Controller SW Unit specifications

Provide a brief description of the Controller functionalities and its interfaces.

## Interfaces

| Name | Unit | Type | Data Type | Dimension | Min | Max |
| --- | --- | --- | --- | --- | --- | --- |
| BrakePedalPressed | Unitless | Input | Boolean | 1x1 | 0 | 1 |
| ThrottlePedalPosition | Unitless | Input | Single | 1x1 | 0 | 1 |
| AutomaticTransmissionSelectorState | Unitless | Input | Enum | 1x1 | 0 | 4 |
| VehicleSpeed_km_h | Km/h | Input | Single | 1x1 | -60 | 240 |
| TorqueRequest_Nm | N*m | Output | Single | 1x1 | -80 | 80 |
| AutomaticTransmissionState | Unitless | Output | Enum | 1x1 | 0 | 4 |

Draw the Finite State Machine (FSM) representing the controller logic

3 - FSM Overview

4 - Left half of FSM

5 - Right half of FSM

Comment on the design choices of the FSM, which are not trivial to be understood just by analyzing the controller logic.

As the transmission selector is “by wire”, some transitions could not be seen. The controller can handle any loss of transitions thanks to appropriate inequalities checks that take into account the values as defined in the enumeration (e.g. AutomaticTransmissionSelectorState<Transmission.Brake to switch from B to D).

Another design choice was to divide the Brake state into two substates:  Nominal_brake, which describes the typical behavior of the item in B mode, and Stopping, that manages the case in which the car is coming to a complete stop after using regenerative braking.

The latter substate implements a proportional control, with a chosen value of the gain Kp, that determines the torque request necessary to bring the vehicle as close as possible to actual zero speed, since the nominal behavior may lead the vehicle to maintain a small, yet negative, speed.

Comment with plots of the results obtained from the test cases (it is suggested to use the Data Inspector)

The reported graph shows a behavior similar to the reference one.

It is possible to see the effect of the Stopping substate in the signal TorqueRequest_Nm at around 47s, where there is a brief positive peak that brings the speed to zero.

As a consequence of a design choice, which strictly follows the given equations for transitions of the state, without further refinements, the AutomaticTransmissionState signal may exhibit subtle differences to the one in different implementations, which can be seen in the first transition in which the state switches, in our controller, to Drive for a few seconds before reaching Brake.

| Usually, the first gear of a car has a transmission ratio about ig=3, hence the engine makes 3 complete rotations very single rotation of the pinion gear of the differential. In the differential (final drive), the pinion gear makes about if=4 revolutions for every revolution of the axle shafts. From here, the typical ratio of it=12 between the torque at the wheel and the torque at the engine/electrical motor. | Figure from http://www.thecartech.com/subjects/auto_eng/Gearbox_ratios3.htm |
| --- | --- |

Input, Output, Local, Global, Volatile

Struct, Double, Integer, Enum, Boolean, etc…

Park(0), Reverse(1), Neutral(2), Drive(3), Brake(4)